# Quality Assurance Documentation and Procedures

(General)

Institution			Date

# Performed

| # QA procedure manual present # Dosimetry data books consistent/present Dosimetry equipment calibrated every 2 years Primary and secondary equipment intercompared Test cases available for new version of TPC software Independent verification of mu calc Port films taken for each patient Brachytherapy source locations noted Brachytherapy sources are secure Leak testing of brachytherapy sources HDR source calibration HDR source positioning check HDR safety interlocks HDR area monitor present HDR survey meter present | #  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No  Yes  No |
| --- | --- |

Comments on any "NO"

# Quality Assurance Documentation and Procedures

(X-rays)

Institution:  		Date:

# Machine:			Energy:

# Performed

## Annual Checks (x-rays)

Full calibration			Yes		No

Documentation of links between annual and monthly output checks			Yes		No

Output factors (Sc,Sp)			Yes		No

CA PDD/TMR constancy			Yes		No

OAR constancy.			Yes		No

Tray factors			Yes		No

Wedge factors (FS and depth dep.)			Yes		No

Monitor chamber linearity			Yes		No

X-ray output vs. gantry angle			Yes		No

OAR constancy vs. gantry angle			Yes		No

Collimator rotation isocenter			Yes		No

Gantry rotation isocenter			Yes		No

Couch rotation isocenter			Yes		No

Coincidence of above isocenters			Yes		No

Tabletop sag			Yes		No

Vertical travel of table			Yes		No

Safety interlocks			Yes		No

Timely annual checks			Yes		No

Comparison to current clinical values			Yes		No

Action level indicated			Yes		No

Documentation of corrective action (if necessary)			Yes		No

Comments on any "NO"

## Monthly Checks (x-rays)				Performed

X-ray output constancy			Yes		No

Backup monitor constancy			Yes		No

CA PDD/TMR constancy			Yes		No

Beam flatness & symmetry			Yes		No

Emergency off switches			Yes		No

Wedge interlocks			Yes		No

Light and radiation field coincidence			Yes		No

Gantry/collimator angle indicators			Yes		No

Wedge/tray positioning			Yes		No

Field size indicators			Yes		No

Cross-hair centering			Yes		No

Treatment couch positioning			Yes		No

Latching of wedges/trays			Yes		No

Jaw symmetry			Yes		No

Field light intensity			Yes		No

Timely monthly checks			Yes		No

Action levels indicated			Yes		No

Documentation of corrective action (if necessary)			Yes		No

Comments on any "NO"

## Daily Checks (x-ray and electrons)				Performed

X-ray output constancy			Yes		No

Electron output constancy			Yes		No

All electron energies checked twice weekly			Yes		No

Localizing laser			Yes		No

ODI			Yes		No

Door interlock			Yes		No

Audiovisual monitor			Yes		No

Timely daily checks			Yes		No

Action level indicated			Yes		No

Physicist check			Yes		No

Documentation of correction action (if necessary)			Yes		No

Comments on any "NO"

# Quality Assurance Documentation and Procedures

(Electrons)

Institution:  		Date:

# Machine:

# Performed

## Annual Checks (electrons)

Full calibration (all energies)			Yes		No

Documentation of links between monthly and annual output checks			Yes		No

Output factor constancy for electron applications			Yes		No

CA PDD constancy			Yes		No

Extended distance factors const.			Yes		No

Electron output vs. gantry angle			Yes		No

Timely annual checks			Yes		No

Comparison to current clinical values			Yes		No

Action level indicated			Yes		No

Documentation of corrective action (if necessary)			Yes		No

Comments on any "NO"

## Monthly Checks (electrons)				Performed

Electron output constancy			Yes		No

CA PDD constancy			Yes		No

Beam flatness & symmetry			Yes		No

Safety interlocks (electron cone interlocks)			Yes		No

Mechanicals (applicator position).			Yes		No

Timely monthly checks			Yes		No

Action levels indicated			Yes		No

Documentation of corrective action (if necessary)			Yes		No

Comments on any "NO"